Project Name: Unlocking the Secrets of Microbially Influenced Corrosion: From Detection to Control Mechanisms

PI name: Assoc Prof Federico Lauro

Reported by Singapore Phenome Centre (SPC),

Lee Kong Chian School of Medicine,

Nanyang Technological University

Analyzed and prepared by: Abigail Thomson

Checked by: Prof Wang Yulan

NMR analysis

1. Method

Sample preparation (n=2)

500 µL of the bacterial culture and control media were aliquoted out into 2ml Eppendorf tubes each. 100 µL of PBS buffer containing 100% D20 was added and then vortexed for 30 seconds. The samples were then transferred to 5mm NMR tubes for analysis.

2) NMR analysis

Instrument: III HD 600 MHz Ascend NMR spectrometer (Bruker), with Shim system with high order shimming (BOSS III), 5mm BBI 600 MHz Z-Gradient high-resolution probe (Standard Probe), Sample Jet with cooling and preheating.

Solvent used: D20

Sample jet temperature: 5°C

Acquisition pulse sequence: 1d NOSEY

Probe temperature: 26.85°C (300 K)

Control

Figure 1: Bacterial Culture vs Control media spectra (from 0.6 ppm to 2.9 ppm)

Experiment Results

Acetic

Acid

Bacterial Culture

Succinic

Acid

Acetic

Acid

Pyruvic

Acid

Lactic

Acid

Lactic

Acid

Formic

Acid

Figure 2: Bacterial Culture vs Control Media spectra (from 3 ppm to 8.5 ppm)

Control

Bacterial Culture

Preliminary Conclusion

- Lactic Acid, Succinic acid, Pyruvic Acid and Formic acid are found in the bacterial culture sample and not in the control media sample.

- Acetic acid is found in both the bacterial culture and control samples.

- All peaks identified are putative and have not been confirmed by 2D NMR.